EAST MIDLANDS CONTROL CENTRE DERBY WORKSTATION TRAINING PLAN

TO BE READ IN CONJUNCTION WITH THE RELEVENT SIGNALBOX SPECIAL INSTRUCTIONS

| Prepared by: |  |
| --- | --- |
| Peter Harrison |  |
|  | Date: 01/12/2022 |
| Reviewed By: |  |
|  | Date: |
| Approved by: |  |
|  | Date: |

| Version | Date | Author | Remarks |
| --- | --- | --- | --- |
| Draft V05 | 31/03/2022 | P Harrison | Addition of training schedule |

Derby Workstation Training Plan

Trainees timetable

Week 1

Monday – DCR/DRS Training/if applicable

Tuesday – Simulator Training Derby Workstation

Wednesday 1100 – 1800

Thursday/Friday 1600 – 2300

Weekly progress with SSM/LOM

Week 2

Monday – Friday 0000 – 0645

Weekly progress with SSM/LOM

Week 3

Sunday 0700 – 1400

Monday – Saturday 2300 – 0600

Weekly progress with SSM/LOM

Week 4

Monday – Wednesday 0500 – 1200 can be extended into Thurs/Fri

Thursday/Friday - Pre pass out assessment, area visits, SBSIs and

workstation and location knowledge, with SSM who signs the workstation.

Weekly progress with SSM/LOM

Week 5

Monday – pass out

Rest of week TBC (Working panels/additional breaks?)

Evidence

| Element | Achieved | Date | Evidence | Signaller on Duty |
| --- | --- | --- | --- | --- |
| T3 |  |  |  |  |
| Matlock Branch |  |  |  |  |
| Arrivals Etches Pk |  |  |  |  |
| Departures Etches Pk |  |  |  |  |
| Arr/Deps RTC |  |  |  |  |
| Visit Derby Station |  |  |  |  |
| Visit Etches Park |  |  |  |  |
| Litchurch Lane |  |  |  |  |
| Chadd sidings/roads |  |  |  |  |
| Sinfin branch |  |  |  |  |

CONTENTS

Section 1 – Area of Control

Section 2 – Train Waiting Berths

Section 3 – Traffic Flows

Section 4 – Signals; Points; TPWS; AWS and Telephones

Signals

Signal prefixes

ESOC – Emergency Signals on Control

POP Groups

Flashing Yellow Aspects

Preliminary Route Indicators

Opposing Lock Omitted (back to back shunt)

Limited Clearance

Split Detected Points

AWS and TPWS*

Telephones

Emergency Communication

Section 5 – Preferred/non-preferred routes

Section 6 – Derby Station

Platform Lengths

Permissive Working

Service Platform

Method of Communication to Station Staff

Section 7 - Etches Park

Section 8 – RTC Sidings

Section 9 – Litchurch Lane

Section 10 – Chaddesden Sidings

Section 11 – Sinfin Branch

Section 12 – Sunnyhill Loops

Section 13 – St Andrew’s Siding

Section 14 – Matlock Branch

Section 15 – Spondon Obstacle Detector (OD) Crossing

Section 16 – User Worked Crossings

Section 16b User worked Red and Green Crossings

Section 17 – Lock Out Devices (separate training plan) Info only.

Section 18 – Westcad Specific Instructions

Section 19 – Hot Axle Box Detectors – HABD

Section 20 – Points in Overlap and signal reversion

Section 21 – Track Section Interrupters

Section 22 – Emergency Replacement Switches

Section 1 – Area of Control

## General

The Derby Workstation has assumed control of the area previously controlled by Derby PSB Centre Panel which has been abolished.

The control of this area is now based in the EMCC utilising WESTCAD as a control system.

DRS/DCR is fitted to this workstation and the method of train detection is by Axle Counter (Except for north of Duffield, which uses the Track Circuit Method of detection and is a re-control)

Axle counters are identified for example PNK

Track Circuits are identified with a (T) for example T123

Train Waiting Berths are identified with a (TW) prefix

NOTE: Only the minority detection system is shown on the screen, IE, on the Nottingham WS the (T) is shown through the station area where detection is by Track circuit.

Track Circuit Block Regulations will apply apart from the Matlock Branch which is Token Block.

Derby Workstation fringes to, Burton Workstation, Trent Workstation and Chesterfield Workstation all based within the EMCC.

## Area Of Control

The Derby Workstation controls the following areas:

Spondon (exc.) and Sunnyhill (exc.) through Derby Station to South Wingfield (exc.) and Matlock

The Derby WS will control access and egress to Chaddesden Sidings, St Andrews Sidings, the Matlock Branch and the Sinfin Branch

The Derby WS will also service Etches Park Depot, Derby RTC and Litchurch Lane (Bombardier site)

Section 2 – Train Waiting Berths

Train Waiting Berths are present on the majority of access from sidings/depots.

As a rule a route cannot be set until the section is occupied by a train.

However from the bypass and F&I lines the routes will go in but the signals will not clear.

From St Andrew’s the route will set and the signal will clear

Apart from exiting Etches Park the signaller will interpose the headcode.

If an axle counter is showing occupied for a prolonged period of time (as it may be a shunt up to the signal) the signaller needs to ascertain whether there is the presence of a train or not.

If there is no train then the axle counter needs to be reset. To reset restore then the prefix is TW-xxx.

Section 3 - Traffic Flows

Passenger trains from the South

It is envisaged that trains to Sheffield and Matlock trains will use platform 5 predominantly, with trains for Birmingham using platform 3 or 4, and obviously there is no access to platform 1 or 2.

Passenger trains from the West

It is envisaged that trains for the North will use platform 1. Trains for Nottingham will use platform 3 or 4 again there is no access to 3 or 4 from the fast lines. The Crewe service will use either platform 3B or 4B for the layover.

Passenger trains from the North

It is envisaged that all passenger trains to the West will use platform 2 and the services from Matlock and Sheffield services (and Northern) will use platform 6, however there is an option to use platform 3 or 4.

Freight trains from the West

Trains that are travelling to the North will use platform 1. However, if a freight train from the West is required to go into Chaddesden sidings then that needs to be signalled via the up or down Tamworth Slow line through platform 3 or 4 to Chaddesden sidings.

Freight trains from the South

For the North may be signalled through platform 3, 4, 5, or the pilot line however obviously the pilot line is a slower option the quickest option is through platform 5, platform 6 is available to run through the station again it’s a slower move.

Freight trains from the North

Trains to the West will run directly through platform 2 there is also an option of using platform 3 or 4.

Freight trains from the North

Trains to the South may be signalled as suggested earlier via platform 3 or 4 or the pilot line however the optimum route is via platform 6.

the regulating of freight trains will be at Sunnyhill, Broadholme or the up slow line at Breadsall, trains can be brought into Derby station and held for regulating purposes in the station only if length applicable but this is not advised.

Trains that have crew relief may have to be held at a regulating point until the signaller is sure that the relief driver is ready at Derby. (This is to be confirmed with the FOC.)

Approach from Burton Workstation

Trains travelling towards Derby are required to be sorted out for the station at DW 5316 signal. It should be noted that there are no routes other than the North from platform 1 or 2 from DW 5316 signal. Trains can be signalled up Tamworth fast down Tamworth fast (platform 1 or 2) Up Tamworth slow or down Tamworth slow (platform 3 or 4). Trains that will reverse towards sheet stores should be signalled either onto the up or down Tamworth slow line. It should be noted that from platform 3 or 4 it is possible to go North via Ambergate, or into Chaddesden sidings, or reverse and access Etches Park via 305 points, Litchurch Lane wagon works via 202 points, or RTC North or South via 309 or 302 points respectively.

Trains coming out of Derby station towards Stenson there is only one route from platform 1 and that is through 201 points reverse onto the down Tamworth fast one route out of platform 2, from platform 3 and 4 the down Tamworth slow and the up Tamworth slow can be accessed via 314 or 315 points.

Trains departing for Stenson on the up or down Tamworth slow line approaching DW 5313 or DW 5311 may receive a single flashing yellow. The line speed through the scissor crossover is 25 mph, the line speed increases to 50 mph and beyond Osmaston Road Bridge increases to 70 miles an hour on the down Tamworth fast. The crossovers beyond Osmaston Road Bridge, numbered 205, 206 and 207 is a 50 mph Junction. The line speed increases again back to 90 miles an hour on the down Tamworth beyond bridge DBP 1 – 5

Approach from Trent Workstation

The extent of the lookback towards Long Eaton is the overlap of TD 4431, which is just before Sawley CCTV crossing.

The strike-in for a non-stop train of Spondon MCB OD crossing is at same place, the limit of indication has moved back towards Long Eaton by 2 signal sections. The strike-in for Spondon MCB OD crossing for a stopping train is the joint between DMB and DMC track sections.

The signal that protects Spondon crossing in the down direction is TD 5029 which is 50m from the crossing. The protecting signal for Spondon crossing in the up direction is TD5032; both signals have automatic working facility.

From Spondon direction the 1st difference that is notable is the new trailing crossover 301 A and B.

On the up main at the toe of 301 A points there is a new signal reading in the down direction TD 5039, this signal is for turn back moves.

There is only one route from this signal and that is along the down main to TD5045.

Note, if a train is signalled towards TD 5040 to make a turn back move from TD5039, TD5040 must be replaced to danger before the movement takes place. This is because the train that is to make the reverse move will be within the striking controls for Spondon MCB OD crossing, having TD5040 “ON” will prevent the crossing initiating. 0S00 must be applied in the TD to advise the Trent WS

Trains signalled in the down direction towards TD 5045 can be routed to platforms 3, 4, 5, 6; they can also be signalled onto the pilot line, and into the service platform. Permissive movements into platform 3, 4, 5, 6, and the service platform are also available from this signal.

Trains that are signalled into platform 3, 4, 5 will get an unrestricted proceed aspect, “main aspect free” (MAF), trains signalled into 4, 6 or the pilot line/service platform will receive a restrictive aspect which is approach released, “main aspect restricted”, (MAR).

Preferred route – check MAF and MAR

There are 2 routes into platform 4 from TD5045 MAF via 303 points normal, which is the preferred route, also available is MAR 303 points reverse this is non-preferred.

When a train is signalled from TD5045 to the pilot line/service platform this is a MAR route which presets EC5477 signal.

TD 5049 the exit signal from the RTC sidings North. This is a 4 aspect signal .Trains from here can be signalled into platform 3 or 4 only .They can be signalled under permissive arrangements. There is also a signal move into the headshunt beyond the signal if required. Trains that are signalled into platform 3 and 4 get MAF

EC5474signal. A ground position light on the pilot siding leads into Etches Park carriage sidings only; this is slotted with the shunter’s release, meaning that it is a single pull signal. This signal also has last wheel replacement.

This signal can also be off with EC5475 allowing shunt movements to take place. (Back-to-back)

EC5475 is a 4 aspect signal. This is the exit signal from Etches Park it controls all movements towards Derby station from Etches Park. From this signal you can signal trains into platforms 3, 4, 6, and the pilot line. Platforms 3, 4, 6 are MAF.

Trains cannot be signalled into platform 5 from this signal.

The signal has a single shot clearance from Etches Park and has last wheel replacement for trains signalled onto the pilot line.

Approach from Chesterfield Workstation/Ambergate Jcn

The limit of the re-signalled area of Derby workstation is 132 ½ miles

The 1st change is that Duffield HABD has moved to Belper sidings this is to allow safe stopping of a train activating the hotbox note an SBSI will be required.

DC 5108 is an automatic signal capable of showing 2 yellow flashing aspects for the diverging routes, DC5106 is a 3 aspect signal, capable showing one flashing yellow aspect. This signal is a distant signal for DC5104 this is a closing up signal in connection with the transition from 3 aspect to 4 aspect signalling.

DC 5104 has 2 Preliminary route Indicators (PRI’s) associated with it.

Trains can be routed to the down fast in the up direction or up fast up direction or up slow up direction from this signal.

Flashing aspects at the previous 2 signals is applicable to the diverging routes; the flashing aspect inhibition point is the section between FEY track and FEV track. The speed through the turnout 354 is 50 miles an hour.

Note, the route from DC5092 and DC5096 must be set for the flashing aspects to work.

DC5102 signal.

This is turn back signal on the down main reading in the up direction, a train can be signalled onto the down slow in the up direction; along the down fast up direction; onto the up fast or up slow both the up direction this signal is capable of calling on into the up slow for run round purposes. The signal has TPWS but no AWS magnet this signal can be used as an end of single line working signal if single line working was implemented over the down main.

Down Slow line.

The down slow line can be used to run round the train that has arrived from the South, engine off forward beyond DC 5102 signalled up fast to behind DC 5093 then signalled back into the down slow line onto the train.

Up Slow Line

The Up Slow Line can be used to run round a train that has arrived from the north. Train is routed to DC5092 and the engine detaches. Engine signalled behind DC 5087 or DC5089 then signalled to DC 5102 on the down fast. Engine is then signalled onto the train, the train then departs towards the North.

DC5099, on the down slow line is not approachable at red, this is a requirement to prevent a “misread” across when approaching this gantry, the “calling on” signal will however allow a train onto an occupied down slow for the purpose of attaching an engine for run round move.

DC5107 is not an approached control signal therefore the train could be signalled onto the down slow and straight through and out at the North end, the speed of the down slow is 40 mph.

A train signal signalled onto the up slow from DC5104 signal has the opportunity of going back onto the up fast at DC5084 with full access to A, B, and C lines at this point, the train may also continue from DC5084 with 348 points normal onto the D line, note, there is no access to A, B, and C line, after this point. A train may be signalled from DC5076 on the D line into platform 3, 4, 5, 6, Derby pilot line and the F and I line.

Run round facility

There is a run round facility for a 775M train on D line/up slow. The train is signalled towards DC5097 engine off and signalled to behind DC5102 the engine would return to Derby station, and signalled from DC5089 onto the rear of the train the train then draws forward to DC5084.

Signalling movements

A train is signalled on the up fast to DC5088 has access to the, C and D lines from this point, there is no access to platform 1 in the wrong direction; all other platforms are available from this point onwards.

A train signalled from DC5104 onto the down fast in the up direction can be signalled back to the up fast at DC5096 or continue to DC5090 where it could be signalled across onto B, C, and D lines or directly into platform 1 in the wrong direction.

Trains signalled out of Derby along the A line which becomes the down fast is the preferred route (only route from platform 1) trains signalled out of Derby to the North, from platform 2 can be signalled back to the down main at DC5085 or DC5095 or finally at DC5103 similarly trains coming out of platform 3 or 4 along C line the fastest route is to signal the train onto the down fast at DC5087 however the train may be allowed to travel along C line through 347 reverse onto the up fast at this point.

Trains coming from D line will be signalled from DC5089 through 344 and 347 points onto the up fast in the wrong direction and then through 349 points onto down fast.

Up & Down Broadholme Loops

These 2 loop lines have passenger line status, and can be used accordingly so, but not permissively if a passenger train is used on the loop. They can be used to regulate early and late passenger trains, and as a refuge for failed passenger trains should the need arise.

When these loops are shortened after the Ambergate Remodelling project the new lengths will be published.

St Mary’s Loops

Insert picture

Section 4 – Signals; TPWS; AWS and Telephones

## Signals

All signals on Derby Centre WS are Dorman searchlight self-faulting LED signals of either 2, 3 or 4 aspects. An example of this signal type is shown below.

Note that only two signal windows are present, this allows all four signal aspects to be displayed, above this particular signal is an LED Route Indicator (RI)

Ground Position Signals are also Dorman type LED and have the line of route prefix.

All signals with a ‘R’ are reliable and can be used for protecting the following:

T3

Line Blockage

Temporary Block Working

Single Line Working

## Use of “Emergency all signals on control (ESOC)” during failure of SSI equipment.

The control may be used to place or maintain signals at danger in an emergency when there is a failure of the electronic link between the screen and interlocking.

However, because of the way the interlocking’s configured, the ESOC control on this workstation will also control signals to danger on the Derby Centre WS.

Predefined Overrun Protection – POP Groups

There are a number of POP groups on the workstation. The details of these are listed in the SBSI’s.

## Signal Prefixes

Signals on the Derby Workstation will be prefixed according to line of route (except the re-controlled section Duffield to the north remains DY prefix)

DW indicates Derby to Wichnor Jcn

TD indicates Trent Jcn to Derby

DC indicates Derby to Chesterfield

EC indicates Etches Park to Chaddesden

Preliminary Route Indicators

There are a number of PRI’s on this scheme, which indicate to the driver the route that he will be going if no flashing aspect are provided, i.e. where there is no reduction in speed at the Junction and the driver gets a green aspect in the Junction protecting signal.

PRI will replicate the Junction indicator position, if the PRI indication fails the signal will become approach control signal, there may be more than one PR I for a signal for example the PRI for DW5316 has two PRI’s named DW 5316PR (A) and DW 5316 PR (B)

Signals with opposing locking omitted.

The following signals can be cleared whilst the opposing signal is also cleared for shunting purposes EC5481/EC5484, EC5483/EC5484, EC5483/EC5480. It should be noted however if these signals are set for shunting the overlap beyond either EC5487 or EC5489 will be occupied. Sils?Sis should be applied.

Flashing Yellow indications have been provided at certain locations to assist crossover speed improvements, a list of these signals is provided in this plan

| FLASHING ASPECT SEQUENCE |  |  |
| --- | --- | --- |
| Signal No | Aspect | Signal Ahead |
| DW5328 | FLASHING DOUBLE YELLOW | DW5316 |
| DW5324 | FLASHING YELLOW | DW5316 |

The signalling principles normally need the route to be set from the first signal beyond the junction for the flashing yellows to flash unless it is a loop where drivers expect to stop or fully comprehensive TPWS is provided.

This will be the case when the route is set to the diverging route.

Banner Repeating Signals

Banner Repeating Signals are associated with the following signals:

DC5101

DY552

The following Banner repeating signal DC5101BR, will be able to display a green banner (white background changes to green background) when the signal to which it applies is displaying a green aspect.

Workstation indications will remain as a standard active icon, with a BR

## Limited Clearance Signal

See SBSI

Split Detected Points

Points on the workstation that have both an ‘A’ end and ‘B’ end are split detected. If an end has lost detection but detection remains in the other end then train may run but the protecting signal will not be able to be cleared. Thus passing the signal at danger.

## AWS &TPWS

AWS inductors located 180 metres on the approach to the main running signals are provided.

AWS is provided at Derby Station platform starting signals.

TPWS is fitted to all junction protecting signals, Signals on BiDi lines and where required PSR’s.

Signals fitted with TPWS are denoted by the signal number being in blue.

PSR’s fitted with TPWS will be denoted by a blue P

NOTE: that if a failure occurs with the TPWS, a blue square will flash alongside the Signal identity, or if the TPWS fails on a PSR, the blue P will flash.

## Telephones

All Main aspect signals capable of showing a Red aspect will be fitted with Signal Post Telephones (SPT).

At signals which are right hand mounted, or gantry mounted signals where the SPT would be in limited clearance, the SPT will be right hand mounted in the Cess. Where this is not possible in multi-track areas and the SPT would be in limited clearance, a white diamond sign with a cross will be provided.

At Derby station, the SPT`s for platform starting signals will be located on the platform.

Telephones will also be provided at Train Ready to Start (TRTS) plungers at Derby station at LOD locations and at Point Zone locations throughout the Workstation.

EMERGENCY COMMUNICATION

GSMR operates over the Derby Centre Workstation

Station Radios

When using the Station radios provided on the Centre panel, strict radio protocol must be used at all times. The radio can be used to contact a member of the Station Staff quickly, but when a safety critical message needs to be passed, it must be done on a landline telephone. The radio call sign numbers are:

Zero One – Station Manager

Zero Two – Platform 1 staff

Zero Three – Platforms 2 & 3 staff

Zero Four – Platforms 4, 5 & 6 staff

Zero Five – Station Reception

Zero six Announcers – Station Announcer located on the station

Zero Seven Signaller – Derby Workstation Signaller

Other numbers are used for cleaners and lost luggage, but are not used by the Centre panel Signaller. When re-platforming a train, ensure that the relevant station staff are advised by radio as soon as you make the decision as they have the general public to manage on the platforms.

Section 5 – Preferred and Non-preferred Routing

From TD5045 to platform 4 there is a preferred and non-preferred route. The route is via 306pts and 311pts. The non-preferred route if 311pts are normal (either locked or by a route set through) then the route is via is via 303pts reverse and 307pts.

Section 6 – Derby Station

Platform Lengths – South/West end to North end

Platform 1

PSL= 144m – PSM=47m – PSU=140m Operational length = 327m (14 single car lengths)

Platform 2

YTW= 131m – YTV= 47m – YTS= 130m Operational length = 308m (13 single car lengths)

Platform 3

NGR= 120m – NGS= 68m – NGU= 117m Operational length = 305m (13 single car lengths)

Platform 4

RJY= 120m – RJW= 68m – RJV= 117m Operational length = 305m (13 single car lengths)

Platform 5

DPG= 120m – DPJ= 54m – DPK= 101m Operational length = 275m (11 single car lengths)

Platform 6

FMP= 133m – FMN= 48m – FML= 130m Operational length = 311m (13 single car lengths)

Service Platform

225m (9 single car lengths)

(The above does not include any sighting distances as these vary platform end to platform end)

## Example Unit Lengths

4 CAR Meridian/Voyager – 92M

5 CAR Meridian/Voyager - 115M

9 CAR Meridian/Voyager - 207M

10 CAR Meridian/Voyager – 230M

HST (8 CAR) – 224M

1 CAR DMU - 23M

2 CAR DMU - 46M

3 CAR DMU - 69M

4 CAR DMU - 92M

LIGHT ENGINE - 23M

The platforms through Derby station have all been split into 3 sections, this is to give the signaller better information as to where trains are stood in relation to any permissive move that may be required, a long train e.g. HST, 10 car, 7 car, of 5 car Meridian may occupy all 3 sections simultaneously, however single car, 2 car, 3 car train would only occupy a single or 2 sections at once this would inform you that there is xx number metres clear at the opposite end of the station as to where the train stood train, indicative lengths of the platform and car lengths will be published in the SBSI in a similar manner to Nottingham.

All of the through platforms 1 to 6 are all passenger permissive with no restriction.

Entry into platform 6 from the North the driver will have to be stopped and advised that they will be signalled from DC 5076 into an occupied platform this is because of signal sighting restrictions. (This does not apply to EC5486/EC5488 on Chaddesden curve)

This will also be published in the signal box instructions and in the sectional appendix. The service platform is only to be used by ECS, without any other special restrictions been in place.

An additional line has been provided called the fuel inspection line, allowing better access to Etches Park, the pilot line extends over the Derwent Bridge towards the Chaddesden curve points coming out of Chaddesden curve also connect into D line.

Section 7 – Etches Park Depot and Carriage Sidings

The working of Etches Park sidings.

The fundamental working of Etches Park sidings will not change.

Etches Park can be accessed by either the Fuel & Inspection Line (F & I Line), the Bypass Line or via the Way and Works Junction entrance. The F&I entrance now consists of 2 lines and entrance via 320 points from the North or via either line from Chad Curve. Trains departing via Way and Works Jn will have the description entered into the TD by the Operations Supervisor and will be controlled either entering or departing by the new Etches Park shunters panel equipment. Trains departing or arriving via the Bypass or F&I lines will be by the traditional method of communications with the Operations Supervisor for acceptance.

On the Bypass Line and Stabling 1&2 there are boards indicating “Stop and wait until illuminated” – associated with EC5481

On F&I lines 1 and 2 there are “Stop, telephone and wait signal ahead” boards – The Operations Supervisor at Etches Park will advise of the headcode.

Etches Park Shunters Panel

The Shunters panel at Etches Park allows free flow outward from EC5475. The panel has to be in the “Open State” with “Departures from EPCS. The shunter will input the headcodes accordingly.

Inbound moves have to be accepted by the shunter as they have to set the hand points appropriately.

The shunter can also request a “Headshunt”

All the above will reduce the amount of comms between both parties

Section 8 – RTC Sidings – To be read with SBSI

RTC sidings North and South.

The RTC sidings method of working will not change from what is now, trains exiting the RTC sidings South can be signalled up to TD 5045 where it can be routed into the station or if it’s a light engine it could be signalled to TD 5045 change ends then signalled from TD 5046 to go South the distance between the 2 signals is approximately 30 m.

Trains coming out of the RTC sidings North from TD 5049 is as stated above.

Access to the RTC North sidings by trains from the North is via platform 3 or 4.

Access to the RTC sidings South can be via the pilot line; platform 6; platform 5; platform 3 or 4.

Trains departing Derby from platform 5 have to run along the down main in the up direction until 301 points whilst this is valid movement it is a slower move.

Section 9 – Litchurch Lane (Bombardier Site) – to be read with SBSI

Trains for Litchurch Lane in or out are via platform 3 or 4. Platform 3, 4 and Chad are gauge cleared to the current flow stock out of Litchurch Lane the North from Derby North Junction to Chesterfield, Barrow Hill and the Erewash Valley has also been gauge cleared.

Section 10 – Chaddesden Sidings – to be read with SBSI

The speed around Chaddesden curve is 25 mph. An additional line has been provided on Chaddesden curve, called the arrival/departure1 and arrival/departure 2

Chaddesden siding’s is fully tracked indicated which includes carriage sidings 1 to 4, storage sidings. Sidings 5, 6 and 7 are non-tracked, there is a run round facility within storage sidings. Hand points control the entrance to this set of sidings. Trains from either EC5491 or EC5493 can be signalled up to the stop board EC5497 stop and check hand points board.

Trains coming out of storage sidings 5 to 7 there is a stop board with the words obtain signals permission to proceed there is one signal that controls the exit from storage sidings 5 to 7 which have 2 routes either arrived/depart one or arrive/depart 2.

Signalling trains into the carriage sidings 1 to 4.

The carriage sidings all have electrical points and are fully signalled, there is an exit signal from each of the 1 to 4 carriage sidings, and there are 2 routes from these signals.

Signal EC5491 also gives access to the outside siding and the outside loop, trains are signalled up to the stop board EC5495 where the Train crew need to check hand points before a train comes off either the outside loop or the outside siding they must contact signaller before moving towards EC5490 (change wording to reflect MOW)

Section 11 – The Sinfin Branch

The Sinfin branch has been sold to Rolls-Royce beyond Sinfin North station. The signal DW5320 is on the platform at Sinfin North Station and will remain a network rail signal maintained by our S &T department.

The line is 15 miles per hour throughout.

Standage in the run round loop on the Sinfin branch is 232 m

Once train is signalled onto the Sinfin branch it will look after itself it is not permitted for the train to approach DW 5320 signal, with another train signal towards the stop and await instruction boards DW 5323.

Find out length between 5323 – 5318?

Section 12 – Sunnyhill Loop

The Sunnyhill loops have an entrance and exit speed of 40 mph and are of passenger status.

DW5332 is a 4 aspect signal capable of showing 2 flashing yellows; DW5330 is a 4 aspect automatic signal capable of showing a single yellow aspect, into the up sunny Hill loop.

Section 13 – St Andrew’s Sidings

St Andrews sidings have been shortened to allow a run round line and a siding line. The entrance to the siding is via 204 points from the up Tamworth fast line. The speed within the sidings is 15 miles an hour. At the entrance to the siding there is an electrically detected spring point (ED) this is denoted by the $ on the screen. If detection is lost on 203 points DW 5316 will not clear into the sidings. It should be noted that 204 points are also restoring points.

All trains will be signalled onto St Andrews siding which is the left-hand line in direction of travel. If a train signalled into St Andrews siding is to run round once in St Andrews siding the engine will proceed to the North end of the siding just under London road bridge beyond the hand points the driver can set the hand points for the run round line and then proceed back towards DW 5309 signal Once changed ends the driver can then proceed onto the St Andrews siding line where you can get onto the rear of the train.

If a train or yellow plant is signalled into St Andrews siding to stable the yellow plant or train must proceed to the North end of the siding beyond London Road Bridge and the hand points and beyond the stop board which states trains must be clear of board. Phone signaller once stabled. Once you have conformation that the train has been stabled and it is north of the stop boards you may then allow a 2nd train into the sidings to run round as described earlier.

Before a stabled train is allowed to proceed into the run round area of the sidings it must pass the stop board with your permission.

Coming out of the St Andrew’s siding the train can draw down to DW 5309 where it will activate the train waiting track. The signal route from St Andrews siding is along the up Tamworth fast in the down direction .The line speed in the down direction is 25 miles an hour this allows a train to proceed towards DW 5321 signal which can show a green aspect without any approach control controls. There are 3 routes from this signal onto the Sinfin branch, into the down Sunnyhill loop, onto the down Tamworth, all main aspects.

Section 14 - The Matlock Branch – to be read with SBSI

The Matlock Branch is operated under the No-Signaller Token system (TS7 of the Rule Book).

The token instrument at Ambergate is released upon occupation of 912 TC in Ambergate Station. Upon obtaining the token, the Driver of a passenger service will obey DY572 signal and proceed along the branch to Matlock and return. RHTT service or charter will possibly transfer to Peak Rail infrastructure at Matlock Station.

On returning to Ambergate Station, the Driver will return the token to the machine at Ambergate and this will allow signalled routes to be set from DY571 signal.

The Ambergate Station token machine

Matlock Ground Frame (above)

See SBSI’s for full instruction.

Section 15 – Spondon (OD) Obstacle Detection Crossing

The strike-in for a non-stop train of Spondon MCB OD crossing is at same place, the limit of indication has moved back towards Long Eaton by 2 signal sections. The strike-in for Spondon MCB OD crossing for a stopping train is this joint between DMB track section and DMC track section.

The signal that protects Spondon crossing in the down direction is TD 5029 which is 50m from the crossing. The protecting signal for Spondon crossing in the up direction is TD5032, both signals have automatic working facility.

Spondon MCB OD crossing is the same as the MCB OD crossing on the Netherfield workstation. The training material is available for MCB OD crossing is in the Netherfield training package. To be provided to staff during the Derby Workstation training for competency.

See other training material

Section 16 – User Worked Crossings

Whatstandwell UWC

The only other crossing equipped with phones is at Whatstandwell on the Matlock Branch. The difference with this crossing to other User Worked Crossings, is that should the token be removed, the user cannot be allowed to cross, as it is uncertain where on the branch the train actually is, but once the train returns to Ambergate Station, then the user can be authorised to cross.

Section 16B- Automatic user worked

Megalaughton Lane Crossing

Automatic user worked level crossing red and green

In the event of crossing going into dark mode than crossing can be treated as a user worked crossing, no need to caution trains.

Section 17 – Lock Out Devices…(INFO ONLY)

The Lockout System

The “lockout” systems provide a safe method of signal protection.

The area protected and instructions for its operation are displayed at each “lockout” instrument.

The “lockout” may be used instead of protection arrangements referred to in Rule Book Module TS1Regulation 13 and Handbook 8.

The person requesting protection must be either a certified “Controller of Site Safety” (COSS) or “Individual Working Alone” (IWA).

Method Of Protection

When a “lockout” system is to be used to provide protection, the following procedure must be observed:-

A.	Taking a “lockout”

a)	The COSS/IWA must telephone the Signaller giving his/her Name, Grade, Employer, location and contact telephone number.

b)	The COSS/IWA must state which “lockout” section they require to be protected by and the time duration.

c)	If use of the “lockout” is agreed, the Signaller must record these details in the train register book and repeat them back to the COSS/IWA who must confirm they are correct.

d)	When satisfied the arrangements are correct, the Signaller may authorise the COSS/IWA to remove the Lockout Key, or operate the lockout.

e)	The COSS/IWA must observe the instructions displayed and remove the lockout key or operate the lockout.

B.	When the protection is no longer required

a)	When the protection is no longer required, the persons working under the control of the COSS must be moved to a position of safety and briefed on the changed protection arrangements.

b)	When the line(s) is/are clear and safe for trains to run on, the COSS/IWA must advise the Signaller of his/her Name, Grade, Employer and location.

c)	When advised to do so by the Signaller, the COSS/IWA must replace the Key, and turn it clockwise until it is locked.

d)	The Signaller must be advised and the COSS/IWA must check that the normal indication has been restored.

e)	The Signaller must advise the COSS/IWA and make an entry in the train register.

C.	Exceptional circumstances

a)	The COSS/IWA who gives up the “lockout” protection, must, except in exceptional circumstances, be the same individual who obtained the “lockout” protection.

b)	In exceptional circumstances, the COSS/IWA may hand over to a relief provided he/she advises the Signaller of the Name, Grade, Employer, location, and contact telephone number of his/her relief.

Activities Requiring Lockout Protection

Any activities requiring lockout protection will require a NR3180 entry. Line blockages with LOD protection will require an authority number and a NR3180.

The LOD (K)

The LOD (K) captive key system prevents all signalled moves into the protected area and also prevents overlaps being set into the protected area.

The LOD (T)

The LOD (T) key enabled system prevents all signalled moves into the protected area and also prevents overlaps being set into the protected area.

The LOD (P)

The LOD (P) key enabled system prevents all signalled moves against the normal direction of traffic.

The LOD (E)

LOD (E) inhibition of movements in both directions

LOD (P) Inhibition Of Bi-Directional Moves

This type of protection prevents all signalled moves against the ‘normal’ direction of traffic enabling staff to utilise lookout protection for a single direction.

1.	Applying Protection with LOD (P)

The sequence of events to apply protection using a LOD (P) device shall be:-

a)	Authorised User contacts signaller for permission to apply the protection against bi-directional moves,

b)	The signaller checks that ’reverse’ direction routes into the Protected Area are cancelled and no trains are signalled within the area in the ‘reverse’ direction.

NOTE The ‘reverse’ direction route does not include the overlap for this purpose.

c)	Authorised User receives permission including authority number,

d)	Authorised User records authority number on record of arrangements for safe system of work as part of Lookout arrangements.

e)	Authorised User inserts key and turns to ‘Operate’ position.

f)	Signaller operates the release and Authorised User presses ‘Patrol’ button.

g)	The Authorised User checks that ‘Traffic’ indication extinguishes, ‘Patrol’ indication illuminates,

h)	Authorised User confirms with signaller that the correct protection has been applied and is displayed on the signaller’s display,

i)	Authorised User turns key to ‘Normal’ position and withdraws key.

2.	Removing Protection with LOD (P)

The sequence of events to cancel protection using a LOD (P) device shall be:-

a)	Authorised User inserts key and turns to ‘Operate’ position,

b)	Authorised User confirms that the correct device is operated and that the ‘Patrol’ indication illuminates,

c)	Authorised User contacts signaller for permission to cancel protection quoting authority number,

d)	Signaller cancels the release and Authorised User presses ‘Traffic’ button – ‘Patrol’ indication extinguishes, ‘Traffic’ indication illuminates,

e)	Authorised User turns key to ‘Normal’ position and withdraws key,

f)	Authorised User confirms with signaller that the correct protection has been

g)	withdrawn.

Interlocking Requirements

a)	all ’reverse’ direction routes (excluding their overlaps) into the Protected Area are normal and free of approach locking, and

b)	no train is currently within the Protected Area and signalled in the ’reverse’ direction.

NOTE These requirements are based on the fact that the work will not affect the operation of train detection within the protected area.

Signallers Equipment for operation

Each section shall have a white roundel to enable the signaller to release the instrument on site.

Operation of the release shall change the roundel to flashing and filled.

When the authorised user has selected patrol mode then the roundel shall change to white steady and filled and the direction arrows for the reverse direction shall be displayed with a dark blue reminder background. (NOTE this colour may change, in discussion with IRL)

Cancellation of a granted release shall change the roundel to flashing and filled until the authorised user selects traffic mode when the reminder background shall also be removed.

The LOD (T)

The LOD (T) Will inhibit all movements within the control area. This type of protection prevents all signalled moves into the Protected Area. Due to the potential work, the protection also prevents overlaps being set into the Protected Area.

NOTE LOD(T) does not inhibit routes within or out of the Protected Area in order to enable signal testing, etc.

Typical trackside instrument – LOD (T)

1.	Applying Protection with LOD (T)

The sequence of events to apply protection using a LOD (T) device shall be:-

a)	Authorised User contacts signaller for permission to apply the protection for a pre-defined area,

b)	The signaller cancels all routes into, or with overlaps within, the Protected area in both directions,

c)	The signaller checks that the Protected Area is clear (all train detection indicating clear), except where maintenance staff have been called to attend a failure of the occupied train detection sections.

d)	Where a train detection section is occupied, the Signaller shall advise the Authorised User.

e)	Authorised User receives permission including authority number.

f)	Authorised User records authority number on record of arrangements form in a similar way to a Line Blockage with disconnection protection.

g)	Authorised User inserts key and turns to ‘Operate’ position.

h)	Signaller operates the release and Authorised User presses ‘Lockout’ button ‘Traffic’ indication extinguishes, ‘Locked out’ indication illuminates,

i)	Authorised User confirms with signaller that the correct protection has been applied,

j)	Authorised User turns key to ‘Normal’ position and withdraws key.

2.	Removing Protection with LOD (T)

The sequence of events to cancel protection using a LOD (T) device shall be:-

a)	Authorised User inserts key and turns to ‘Operate’ position,

b)	Authorised User confirms that the correct device is operated and that the ‘Locked out’ indication illuminates,

c)	Authorised User contacts signaller for permission to cancel protection quoting authority number,

d)	Signaller operates ‘Traffic’ release cancel control and Authorised User presses ‘Traffic’ button – ‘Locked out’ indication extinguishes, ‘Traffic’ indication illuminates,

e)	Authorised User turns key to ‘Normal’ position and withdraws key,

f)	Authorised User confirms with signaller that the correct protection has been withdrawn.

Interlocking Requirements

The transition from ‘Traffic’ to ‘Locked out’ shall only be permitted if all routes (including routes with overlaps in the area) into the Protected Area have been cancelled and any approach locking released.

When the protection is in place, routes required to be normal to obtain protection shall be prevented from setting.

The interlocking shall require the signaller and Authorised User to operate the release at the same time.

NOTE The subsequent clearance of routes within and leading out of the Protected Area is permitted where require for the works.

Signallers Equipment for operation

Each section shall have a white roundel to enable the signaller to release the instrument on site.

Operation of the release shall change the roundel to flashing and filled.

When the authorised user has selected lockout mode then the roundel shall change to white steady and filled and the track sections within the protected area shall be displayed with a dark blue reminder background. (NOTE this colour may change, in discussion with IRL)

Cancellation of a granted release shall change the roundel to flashing and filled until the authorised user selects traffic mode when the reminder background shall also be

removed.

The LOD (K)

The LOD (K)  – Protection system for blocking entry to a section of line(s) A LOD (K) system inhibits moves into the Protected Area in both directions. Due to the potential work, the protection also prevents overlaps being set into the Protected Area.

NOTE The operation of the LOD(K) device may be used to establish a green zone.

NOTE LOD(K) does not inhibit routes within or out of the Protected Area in order to enable signal testing, etc.

Typical trackside instrument – LOD (K)

1.	Applying protection with LOD (K)

The sequence of events to apply protection using a LOD (K) device shall be:-

a)	Authorised User contacts signaller for protection.

b)	Signaller cancels all routes into (including overlaps), through or out of Protected Area.

c)	The signaller checks that the Protected Area is clear (all train detection indicating clear), except where maintenance staff have been called to attend a failure of the occupied train detection sections,

d)	Signaller operates release device,

e)	Authorised User observes ‘Free’ indication,

f)	Authorised User operates ‘Release’ press button and withdraws key,

g)	Signaller observes normal indication extinguished and releases release

h)	device.

2.	Removing protection with LOD (K)

The sequence of events to cancel protection using a LOD (K) device shall be:-

a)	Authorised User contacts signaller advising that the protection is no longer

b)	required,

c)	Authorised User restores key to instrument and turns it to locked position,

d)	Signaller cancels release and observes normal indication.

Interlocking Requirements

The lockout device shall be proved normal (key(s) in instrument(s) and locked in position) for both route setting and aspect controls for:-

a) Routes into the Protected Area, and

b) Routes outside the Protected Area where the overlap extends into the Protected Area.

NOTE Where an overlap extending into the protected area has timed out then the protection may be established.

The interlocking shall only enable the signaller’s release when all routes which will be locked out by the LOD(K) are normal and free of approach locking. The signaller shall be responsible for ensuring that there are no trains in the Protected Area.

The normal indication on the signaller’s display shall only be illuminated when all the key(s) are locked in the instrument(s).

Signallers Equipment for operation

Each section shall have a white roundel to enable the signaller to release the instrument on site. Operation of the release shall change the roundel to flashing and filled.

When the authorised user has removed the key, the roundel shall change to white steady and filled and the track sections within the protected area shall be displayed with a dark blue reminder background. (NOTE this colour may change, in discussion with IRL)

Cancellation of a granted release shall change the roundel to flashing and filled until the authorised user restores the key when the reminder background shall also be removed.

LOD (E) – Protection system for Railway Undertakings

A LOD (E) system inhibits moves in both directions on a section of line including moves into and out of the Protected Area.

NOTE Where permitted by the Rule Book or local instructions, the operation of the LOD(E) device may establish a green zone.

8.1 Applying protection with LOD (E)

The sequence of events to apply protection using a LOD (E) device shall be:-

a) Authorised User contacts signaller for protection,

b) Signaller cancels all routes (including overlaps) into, through or out of the protected Area,

c) Signaller operates release device,

d) Authorised User observes ‘Free’ indication,

e) Authorised User operates ‘Release’ press button and withdraws key,

f) Signaller observes normal indication extinguished and releases release device.

Removing a Lockout with LOD (E)

The sequence of events to cancel a protection using a LOD (E) device shall be:-

a) Authorised User contacts signaller advising that the protection is no longer required,

b) Authorised User restores key to instrument and turns it to locked position,

c) Signaller cancels release and observes normal indication.

Interlocking Requirements

The lockout device shall be proved normal (key(s) in instrument(s) and locked in position) when the release is cancelled. The absence of release shall be proved for both route setting and aspect controls for:-

a) Routes into the Protected Area,

b) Routes originating within the Protected Area,

c) Routes outside the Protected Area where the overlap extends into the Protected Area.

The interlocking shall only enable the signaller’s release when all routes which will be locked out by the LOD(E) are normal and free of approach locking.

The interlocking shall require the signaller and Authorised User to operate the release at the same time for the key to be released.

The normal indication on the signaller’s display shall only be illuminated when the release has not been granted.

NOTE The protection may need to be taken around a train, hence the controls shall be arranged to permit this.

NOTE The Protected Area may cover more than one line to enable a train on one line to be

examined from the adjacent line.

When the protection is in place, routes required to be normal to obtain protection shall be prevented from setting.

VDU based system

Each section shall have a white roundel to enable the signaller to release the instrument on site. Operation of the release shall change the roundel to flashing and filled.

When the authorised user has removed the key, the roundel shall change to white steady and filled and the track sections within the protected area shall be displayed with a dark blue reminder background.

Cancellation of a granted release shall change the roundel to flashing and filled until the authorised user restores the key when the reminder background shall also be removed.

Section 18 – Westcad Specific Instructions

Full training is provided for Westcad operation and use.

Section 19 - Hot Axle Box Detectors - HABD

The HABD has been re-sited at Belper old ground frame. The following signal to be used to bring trains to a stand is – DC5104 at Breadsall.

Section 20 – Points in overlaps and signal reversion

In accordance with NR/L2/SIG/30009/E450 points within the overlap are not normally detected, this has been introduced primarily to remove the need to talk-by two signals for a single point detection failure. However this does not remove the need to prove that overlap track sections are clear. Hence foul tracks are tested if the points are not detected in the Normal position. This introduces a potential change of aspect if a train is called-by a signal and occupies the foul track section. In order to highlight these to the signaller the instances where this could occur are detailed below:

1. 348 Points – Facing beyond DC5084

With 348 points detection failed DC5092A(M) route could be set, talking by DC5085, DC5087, DC5089 or DC5088 will cause a change of aspect on DC5092 signal when foul track FBE becomes occupied. Therefore signaller should replace DC5092A(M) before talking by a movement that would occupy FBE track section.

2. 323 Points – Facing beyond EC5487

Similar to above signaller should replace EC5475E(M), EC5477A(S), EC5481A(S), EC5483A(S), EC5483B(S) and TD5045E(M) before talking by a movement that would occupy FDY track section.

3. 314/315 Points – Facing/Trailing beyond TD5054

Similar to above signaller should replace DC5076E(M), DC5078B(M), EC5486E(M) and EC5488E(M) before talking by a movement that would occupy NGM track section.

4. 315/314 Points – Facing/Trailing beyond TD5053

Similar to above signaller should replace DC5076F(M), DC5078C(M), EC5486F(M) and EC5488F(M) before talking by a movement that would occupy RKA track section.

5. 303, 305 & 306 Points – beyond TD5045

Similar to above signaller should replace TD5037A(M), TD5039A(M) and TD5043A(S) before talking by a movement that would occupy FPB or FMY track sections.

6. 201 Points – Facing beyond DW5301

Similar to above signaller should replace DC5082A(M) before talking by a movement that would occupy YVB track section.

7. 201 Points – Facing beyond DW5304

Similar to above signaller should replace DW5316C(M) before talking by a movement that would occupy PSH track section.

8. 205 Points – Facing beyond DW5311

Similar to above signaller should replace TD5053F(M) and TD5054F(M) before talking by a movement that would occupy NGB track section

9. 207 Points – Facing beyond DW5316

Similar to above signaller should replace DW5318A(M), DW5322A(M) and DW5324A(M) before talking by a movement that would occupy YVL track section

10. 209 Points – Facing beyond DW5321

Similar to above signaller should replace DW5309A(M), DW5311B(M), DW5313B(M), and DW5315B(M) before talking by a movement that would occupy YVS or YVU track sections.

Section 21 - Track Section Interrupters

| Location | Track Section |
| --- | --- |
| 210B points – Sinfin Branch | YVU |
| 204A points – St Andrew’s Siding | PSC(TW) PSB? |
| 202A points – Litchurch Lane | RKE |
| 302A points – RTC South | DMV |

Track section interrupter on the above points, if the section interrupter is struck/displaced and then interrupter activates an audible alarm is received and associated alarm on the GPD. The corresponding track section in the table will show occupied.

You will be unable to reset or restore this track until the interrupter has been reset by the S&T.

If the section interrupter has been activated, trains must not be signalled over until it is established that the line through the points is clear – if necessary by examination by a train.

Once established that the line is clear then trains may run at caution until the S&T attend.

The first train after will be cautioned under Non-Provision of aspect restriction.

Section 22 emergency replacement switches

The following signals are fitted with Emergency Replacement facilities. For note – these signals are not to be used to protect line blockages or engineering works.

DY574, DY583

Appendices

Drivers Handbook/Video